SECTION 11 68 33
ATHLETIC FIELD EQUIPMENT

PART 1 - GENERAL

  - SUMMARY

A. Furnish and install athletic equipment as indicated on the drawings and specified herein, including all labor, materials, equipment, and accessories required for complete installation.

1.2 RELATED WORK

A. Examine Contract Documents for requirements that affect work of this section. Other specification sections that directly relate to work of this section include, but are not limited to: (Note: Edit per your project)

- Section [XXXXXX] – [Related Work Title].

- Section [XXXXXX] – [Related Work Title].

1.3 REFERENCES

A. Comply with applicable requirements of the following standards:

- ASTM International Standards for materials and installation.

- Manufacturer’s published data and recommended installation instructions.

- Local, state, and federal building codes.

1.4 SUBMITTALS

A. Product Data:

- Provide manufacturer’s product specifications, including materials, dimensions, and finishes.

B. Shop Drawings:

- Submit detailed installation and foundation plans for review.

C. Closeout Submittals:

- Submit warranties, maintenance instructions, and as-built drawings.

1.5 QUALITY ASSURANCE

A. Use only pre-approved manufacturers or products meeting specified requirements.

B. Manufacturers and installers shall have a minimum of five (5) years of experience with comparable installations.

1.6 WARRANTY

A. Provide a manufacturer’s warranty covering defects in materials and workmanship for a period of one (1) year from the date of substantial completion.

1.7 DELIVERY, STORAGE, AND HANDLING

A. Inspect all materials upon delivery for damage or defects. Report issues to the Owner’s Representative immediately.

B. Store materials in a clean, dry environment, elevated above ground and under protective cover.

C. Handle materials carefully to prevent damage, following manufacturer’s recommendations.

PART 2 - PRODUCTS

2.1 MANUFACTURERS

A. Basis of Design:

- Products specified in this section are manufactured and/or supplied by:
Beacon Athletics
Address: 901 Deming Way, Ste 101, Madison, WI 53717
Phone: (800) 747-5985
Fax: (608) 836-0724
Email: bids@beaconathletics.com
Website: www.beaconathletics.com

B. Substitutions:

- Substitutions may be considered only if they meet the requirements specified herein and are approved by the Architect or Owner’s Representative prior to bid submission.

2.2 PRODUCTS

A. TUFFY® Windscreen

- Basis of Design: Beacon Athletics - TUFFY® Windscreen

- Contact Information: Beacon Athletics| bids@beaconathletics.com | (800) 747-5985

B. Design and Materials

- Construction: 1,000 denier x 800 denier Vipol® Matrix Mesh (18 x 14 ends/inch)

- Fabric Weight: 10 oz./sq. yd

- Shading: 78% shading

- Tensile Strength: 360 x 320 lbs.

- Sewn Hems: Three-ply hem with ends and corners sewn and finished with two rows of lock-stitched thread

- Thread: High heat bonded polyester with UV inhibitors built into the yarn

- Grommets: #2 brass grommets every 12”

- Seams: 6’ Screens: Solid panel with no horizontal seams. 9’ Screens: Two panels prayer seamed together with a reinforced weld and one row of black UV-treated thread at the center of the screen, with grommets every 12”

- Color: To be selected by Architect or Owner from standard manufacturer colors

PART 3 - EXECUTION

3.1 EXAMINATION

A. Verify that areas and conditions for equipment installation are ready and suitable for work to proceed.

B. Notify the Contractor of any conditions that may impede proper installation. Do not proceed until such conditions are corrected to the satisfaction of the Architect, Engineer, or Owner’s Representative.

3.2 INSTALLATION

A. Install all athletic equipment in accordance with the manufacturer’s written instructions and as indicated in the drawings.

B. Ensure equipment is securely fastened, properly aligned, and fully operational.

3.3 ADJUSTING AND CLEANING

A. Adjust equipment for proper operation as required.

B. Clean all exposed surfaces and remove debris, packaging, and waste materials from the site.

3.4 PROTECTION

A. Protect installed equipment from damage during construction and until final acceptance by the Owner.

END OF SECTION